# LEVEL 0: The Interface and Physical Base

AIM:

- Program the OLED display to act as a menu-driven interface, using the Joystick's Y-axis as a cursor to scroll through options, Push Button 1 to Select, and Push Button 2 to Go Back.

- Create your first menu option: 1. Control Drill. When this mode is selected, you will take manual control of the arm.

- The Stepper Motor acts as the depth winch; read the Joystick's Yaxis to step the motor up (retract) or down (descend).

- The Servo Motor acts as the drill's tilt mechanism, controlled dynamically by sweeping the Joystick's X-axis.

- Throughout this operation, the OLED must function as a telemetry dashboard, dynamically displaying the real-time Depth (Stepper position), Tilt Angle (Servo position), and the current directional status ("Upwards”, "Downwards", ”stationary”).

Constraints:

- The delay () function is strictly prohibited.

- The "Select" and "Go Back" push buttons must use interrupts.

# Circuit:

# Interfacing:

## Joystick:

Source: https://docs.wokwi.com/parts/wokwi-analog-joystick

| Position | HORZ | VERT |
| --- | --- | --- |
| Top-Left | 1023 | 1023 |
| Top | 512 | 1023 |
| Top-Right | 0 | 1023 |
| Left | 1023 | 512 |
| Centre | 512 | 512 |
| Right | 0 | 512 |
| Bottom-Left | 1023 | 0 |
| Bottom | 512 | 0 |
| Bottom-Right | 0 | 0 |

| Name | Description |
| --- | --- |
| VCC | Positive power supply |
| VERT | Vertical axis output (analog) |
| HORZ | Horizontal axis output (analog) |
| SEL | Push button |
| GND | Ground |

- Instead of taking input values at the boundary like 1023 and 0 I took a range so itll be smoother >1000
and <100.

- I’m using the button on the Joystick as my Select botton and another button for back.

## OLED - Display:

Source:https://github.com/mcdominik/source/blob/main/arduino/oled_tutorial/oled_text_example.ino

- It uses I2C Protocol

- I have worked with an LCD screen before this is similar to it

## Stepper Motor:

Source: https://docs.wokwi.com/parts/wokwi-a4988

I’m using the stepper motor with a stepper motor driver(A4988), as there is no battery in Wokwi and it simulates properly even if the voltage or current isn’t correct, I don’t need a battery to run the Stepper Motor. But normally the battery is connected to the motor driver.

| Name | Description | Default * |
| --- | --- | --- |
| ENABLE | Enable pin, active low (pulled down) | Low (0) |
| MS1 | Microstep select pin 1 | Low (0) |
| MS2 | Microstep select pin 2 | Low (0) |
| MS3 | Microstep select pin 3 | Low (0) |
| RESET | Reset pin, active low (floating) |  |
| SLEEP | Sleep pin, active low (pulled up) | High (1) |
| STEP | Step input, connect to microcontroller |  |
| DIR | Direction input: 0=counterclockwise, 1=clockwise |  |
| GND | Ground |  |
| VDD | Logic power supply |  |
| 1B | Connect to motor's B- |  |
| 1A | Connect to motor's B+ |  |
| 2A | Connect to motor's A+ |  |
| 2B | Connect to motor's A- |  |
| VMOT | Motor power supply, not used in the simulation |  |

| MS1 | MS2 | MS3 | Operation mode | Degrees | Microsteps/revolution |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | Full step (default) | 1.8 | 200 |
| 1 | 0 | 0 | Half step | 0.9 | 400 |
| 0 | 1 | 0 | 1/4 step* | 0.45 | 800 |
| 1 | 1 | 0 | 1/8 step* | 0.225 | 1600 |
| 1 | 1 | 1 | 1/16 step* | 0.1125 | 3200 |

- I’m taking MS1 = 0, MS2 = 0, MS3 = 0, for my simulation.

## Servo Motor:

Source: https://blog.wokwi.com/learn-servo-motor-using-wokwi-logic-analyzer/

I tried to do this without libraries with this logic but in wokwi

- 1ms – 0deg

- 1.5ms – 90deg

- 2.4ms – 180deg

So, the mapping wasn’t proper, so I left it.